MA12 column array loudspeaker

ARCHITECTS’ & ENGINEERS’ SPECIFICATIONS

March 2026

The loudspeaker shall be a multiple driver system with matched active equalization as follows.

The transducer complement shall consist of twelve (12) 2.25-in (57 mm) mid/high-frequency drivers mounted in vertical column array. Each driver shall have a rated impedance of 6.2 ohms and wired in series/parallel, resulting in a composite nominal impedance of 8 ohms.

The nominal horizontal beamwidth of the loudspeaker shall be 145 degrees, and the vertical coverage is a band of constant height. The power handling capacity of the loudspeaker shall be 300 watts continuous pink noise, band-limited from 155 Hz to 12 kHz (±3 dB).

The loudspeaker shall be provided with a ducted vent system, tuned at 150 Hz. The input connector of the loudspeaker module shall consist of two parallel-wired NL4 Neutrik speakON connectors and one (1) two-conductor barrier strip.

The enclosure of the loudspeaker module shall be extruded-aluminum sidewalls with steel top and bottom endcaps, all powder-coated. Its outer dimensions shall be 984 mm × 105 mm × 128 mm (38.8 in × 4.2 in × 5.0 in). Its weight shall be 8.5 kg (19 lb).

The loudspeaker shall be the MA12 modular line array loudspeaker.