MA12EX column array loudspeaker

ARCHITECTS’ & ENGINEERS’ SPECIFICATIONS

March 2026

The loudspeaker shall be a multiple-driver, full-range system with matched active equalization as follows.

The transducer complement shall consist of twelve (12) 2.25-in (57-mm) high-excursion, weather-resistant drivers mounted in a vertical column array loudspeaker design. Each driver shall have a rated impedance of 5 ohms and wired in series/parallel, resulting in a composite nominal impedance of 8 ohms.

The nominal horizontal beamwidth of the loudspeaker shall be 160 degrees, and the vertical coverage is a band of constant height. The power handling capacity of the loudspeaker shall be 150 watts continuous pink noise, band-limited from 75 Hz to 13 kHz (±3 dB).

The loudspeaker shall be provided with a ducted vent system, tuned at 75 Hz. The input connector of the loudspeaker module shall consist of two parallel-wired NL4 Neutrik speakON connectors and two (2) two-conductor barrier strips.

The enclosure of the loudspeaker module shall be extruded-aluminum sidewalls with zinc-plated steel top and bottom endcaps, all powder-coated. Its outer dimensions shall be 985 mm × 104 mm × 139 mm (38.8 in × 4.1 in × 5.5 in). Its weight shall be 9.4 kg (21.0 lb).

The loudspeaker shall be the MA12EX modular line array loudspeaker.