# DesignMax Luna DML88P pendant loudspeaker

ARCHITECTS' & ENGINEERS' SPECIFICATIONS

NOVEMBER 2025

The loudspeaker shall contain an 8-inch (203 millimeters) woofer and eight 2-inch (51 millimeters) mid-high drivers mounted around the woofer in a circular ring array.

The loudspeaker shall meet the following performance specifications: On-axis system frequency response shall be 50 Hz to 15 kHz (-3 dB) with the use of recommended active equalization. The loudspeaker sensitivity shall be 85 dB SPL in whole-space environment with Bose Professional EQ voicing with 1 W input at 1 meter. The long-term power handling rating shall be 140 W (AES test methodology using IEC system noise, 2-hour duration). Maximum continuous output shall be 107 dB SPL and the maximum peak output shall be 113 dB SPL, both in whole­space environment with Bose Professional EQ voicing. The nominal coverage pattern shall be 140° conical at 1–4 kHz.

The loudspeaker shall be constructed of an engineered-plastics baffle and enclosure. The loudspeaker shall consist of plastic materials that are resilient to cooking oil exposure. The loudspeaker shall be in compliance with the following safety standards; UL 1480A. The transducers shall be located behind a perforated steel grille that is rotatable and has a powder­coated finish. The loudspeaker shall be available in black or white and shall be paintable. Input connectors shall be a 6-pin Euroblock connector with loop-through, mounted on top. The loudspeaker shall have a nominal rated impedance of 8 ohms and shall be wired in parallel with a line voltage matching (stepdown) transformer with a level selector appropriate for output taps of 12, 25, 50, 100 W and bypass (8 ohms). The loudspeaker input connections shall allow for direct connection to 70-volt, 100-volt or low-impedance amplifiers. The loudspeaker shall include a suspension cable. The cable shall be adjustable with a length of approximately 4.57 meters (15.0 feet).

Loudspeaker dimensions shall be 308 × 492 millimeters (12.1 × 19.4 inches) and net weight shall be 10.0 kilograms (22.0 pounds) with grille.

The warranty shall be 5 years.

The loudspeaker shall be the DesignMax Luna DML88P pendant loudspeaker.